Worksheet: overglaze screen-printing

Materials and tools:

Printing patterns, scizzors, overglaze items

Introduction:

Screen- printing is decorative technique used to apply ceramic colors on the glost fired products. Silk screen printing is the application of ceramic colors by screening through a mesh onto a special paper with a layer which, after soaking, will allow color to be released and transferred to the product.

Instructions: work with printing patterns/stickers

- Choose the pattern – motifs

- Cut them out

- Choose the item – suitable size and shape

- Soak it with water

- Apply it on the surface with the colours up and remove the transfer paper carefully

- Smooth and dry

- Wash the product an hour later (after drying the stickers)

- Fire at 800 °C

Important notes:

The stickers must not be applied one after another, with right side down or leave air bubbles under stickers.